⚡

EV SYSTEMS

VISUAL STUDY GUIDE & ASSESSMENT PREP

Batteries · Motors · Power Electronics · Battery Assessment & Circularity

No prior electrical-engineering background required. Every concept is built from a plain-language explanation → a diagram → a worked example.

# Contents

# 1. The Big Picture — How Power Flows Through an EV

An EV has three separate voltage "worlds" that all talk to each other. Think of it like three rooms in a house, each running on a different power level, connected by translators (converters).

| SIGNAL LEVEL 1V – 12V (sensors, logic chips) | → | 12V AUXILIARY 12V – 27V (lights, control units) | → | HIGH VOLTAGE 27V – 800V (traction battery — drives the car) |
| --- | --- | --- | --- | --- |

KEY IDEA The 12V auxiliary battery still exists in an EV. It powers the low-voltage "brain" circuits even when the big high-voltage battery is switched off, and is kept charged from the main battery through a DC-DC converter.

## Core building blocks

| Block | What it does |
| --- | --- |
| BMS (Battery Management System) | The "brain" — monitors and protects the battery pack |
| Battery Pack → Module → Cell | The physical hierarchy of energy storage (Section 2) |
| DC Converter — Buck | Steps voltage DOWN |
| DC Converter — Boost | Steps voltage UP |
| On-Board Charger (OBC) | Converts AC from the wall into DC to charge the battery |
| Regen (regenerative braking) | Turns braking motion back into electricity |
| EVSE | The charging station/wallbox — supplies AC (slow charging) |
| EVCE / DC Fast Charger | Supplies DC directly to the battery — much faster |
| V2G / V2L | Car sends power back — to the grid (stabilizes it) or to power devices |

ICE vs EV — THE CORE DIFFERENCE An ICE car burns fuel on demand to make power. An EV stores power chemically ahead of time and draws it out electrically. Everything else in this guide follows from that one difference.

# 2. Cells → Modules → Battery Packs

This is the fundamental hierarchy of every EV battery — everything else in this guide sits on top of it.

| CELL (basic unit) | → | MODULE (group of cells) | → | BATTERY PACK (group of modules) |
| --- | --- | --- | --- | --- |

## Cell chemistries at a glance

| Chemistry | Nominal V | Empty → Full | Character |
| --- | --- | --- | --- |
| Lead Acid | 2V/cell | 1.8V → 2.4V | Electrolyte = sulphuric acid |
| Li-ion NMC (Nickel Manganese Cobalt) | 3.7V | 2.8V → 4.2V | Highest energy density; cobalt often from Congo |
| LFP / LiFePO4 | 3.2V | 2.5V → 3.65V | Lower thermal-runaway risk — safer |
| Lithium Titanate (LTO) | 2.4V | — | Very long cycle life |
| Sodium-ion | 1.5–3.0V | — | More cycles, cheaper, less volatile — grid storage |

TRADE-OFF TO REMEMBER NMC packs more energy per kg (great for driving range). LFP is more thermally stable (safer, longer life). This exact trade-off is what you weigh when choosing cells for a second-life storage product.

## Reading a cylindrical cell code — e.g. "18650"

| 1 8 | 650 | 18 = Diameter in mm 65 = Length in mm 0 = Cylindrical shape code |
| --- | --- | --- |

Other formats: Prismatic (rectangular/boxy) and Pouch (flat, flexible — used by Nissan).

# 3. Series vs Parallel — the Single Most Important Concept

| Wiring | Increases | Stays the same | Example |
| --- | --- | --- | --- |
| Parallel | Capacity (Ah) | Voltage | Two 2V/20Ah cells → 2V, 40Ah |
| Series | Voltage | Capacity (Ah) | Two 2V/20Ah cells → 4V, 20Ah |

## Parallel — voltage stays, capacity adds

| 2V 20Ah | + | 2V 20Ah | = | 2V 40Ah |
| --- | --- | --- | --- | --- |

## Series — voltage adds, capacity stays

| 2V 20Ah | + | 2V 20Ah | = | 4V 20Ah |
| --- | --- | --- | --- | --- |

KEY IDEA Energy stored is the same either way: Watt-hours (Wh) = Amp-hours (Ah) × Voltage (V). Both examples above = 80 Wh, whether wired in series or parallel — wiring changes the shape of the power delivery, not the total energy.

## Worked example

- A 12V, 70Ah battery powers a 60W bulb.

- Current drawn = Watts ÷ Voltage = 60W ÷ 12V = 5 Amps

- Capacity test (while discharging) = hours run × current drawn = Amp-hours delivered

- Total energy = Amp-hours × Voltage → e.g. 11Ah × 12V ≈ 132 Wh

WATCH OUT Always double-check whether a spec sheet is in Ah or mAh before you multiply — this is the #1 source of calculation errors.

# 4. Capacity, C-Rating & Energy Density

## C-Rating — how fast can a battery be safely charged/discharged?

FORMULA C-rating = Current drawn ÷ Capacity. Example: a 100Ah battery discharged at 10A → 10A ÷ 100Ah = 0.1C

| Name | Meaning | Typical use |
| --- | --- | --- |
| C100 | Slow discharge over 100 hours | Lead-acid backup |
| C20 | Standard discharge rate | General reference rating |
| C10 | Faster discharge over 10 hours | Solar / lead-acid systems |

| Chemistry | Typical max continuous C-rate |
| --- | --- |
| Lead acid | ~0.2C |
| Lithium | ~0.5C |

RULE OF THUMB High current draw = less usable capacity. Batteries deliver less total energy the faster you discharge them.

## Energy Density — how much energy per kg / per litre?

FORMULA Gravimetric energy density = Energy (Wh) ÷ Weight (kg). Volumetric density = Energy (Wh) ÷ Volume

| Chemistry | Typical energy density |
| --- | --- |
| NMC | 150 – 250 Wh/kg |
| LFP (LiFePO4) | 90 – 100 Wh/kg (lower, but safer & longer life) |

## ★ Worked example: spotting a fake battery

This is exactly the kind of check used in a genuine-vs-fake battery assessment: measure real capacity and weight, calculate the true energy density, then compare against what's physically possible for that chemistry.

| Cell | Claimed capacity | Voltage | Weight | Measured Wh | Wh/kg | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| Cell A | 3000 mAh | 3.7V | 44.35 g | 11.1 | 250 | ✔ GENUINE — within realistic NMC range (150–250) |
| Cell B | 7800 mAh | 3.7V | 30.56 g | 28.86 | 810 | ✘ FAKE — no real lithium cell reaches ~810 Wh/kg |
| Cell C | 1600 mAh | 3.7V | 41.45 g | 5.92 | 142 | ✔ GENUINE — realistic, on the lower end |

WHY THIS MATTERS Real-world lithium cells top out around 250–300 Wh/kg. If a battery's advertised mAh, combined with its actual weight, implies a wildly higher energy density than that, it is almost certainly a counterfeit or mislabeled cell — capacity is inflated on the wrapper while the physical cell inside cannot hold that much energy. Weight + measured Wh/kg is one of the fastest genuine-vs-fake checks available.

# 5. Battery Safety — Thermal Runaway

| Loose connection | → | Overheating | → | FIRE |
| --- | --- | --- | --- | --- |

## Inside a cell — the runaway chain reaction

Raised temperature

↓

Cell becomes more chemically reactive

↓

More heat generated

↓ (self-reinforcing)

THERMAL RUNAWAY — FIRE

WHY LFP IS SAFER This chain reaction is exactly why LFP chemistry (less prone to thermal runaway) is often chosen over NMC for stationary/safety-critical storage, even though it holds less energy per kg.

## Other safety terms

- High-voltage leakage / isolation testing — confirms HV circuits are properly insulated from the vehicle chassis (V = IR, so leakage current reveals a breakdown in isolation)

- DTC (Diagnostic Trouble Code) — fault codes reported by the BMS

- Interlock check — inspect for loose connections on high-voltage circuits before working on them

# 6. Motors — How the Car Actually Moves

## Two main motor types

| Type | How it works |
| --- | --- |
| Permanent Magnet Motor | Neodymium permanent magnets + stator coils. Simple, efficient — most common in EVs |
| Induction Motor (Rotor-based) | AC current creates a rotating magnetic field → induces current in the rotor → rotor spins. Used by Tesla |

## Power flow to the motor

| Traction Battery (HV, DC) | → | Inverter (DC → AC) | → | Motor (spins) | → | Wheels |
| --- | --- | --- | --- | --- | --- | --- |

KEY IDEA The Inverter converts DC (battery) to AC (motors need AC to create a rotating field). The 12V auxiliary battery separately powers the low-voltage control circuit managing all of this.

## Regenerative Braking — running the motor backwards

Car's motion (kinetic energy)

↓

Wheels spin the motor — it now acts as a generator

↓

Magnets spinning past coils generate electricity

↓

Electricity flows back into the traction battery

WHY IT WORKS Same physics as a generator, just run in reverse: normally current + a magnetic field creates motion (motor); here, motion inside a magnetic field creates current (generator).

# 7. Electrical Formulas You'll Actually Use

| Formula | Meaning |
| --- | --- |
| V = IR | Voltage = Current × Resistance (Ohm's Law) |
| I = V / R | Current = Voltage ÷ Resistance |
| Power = VI | Power = Voltage × Current |
| Power = I²R | Power = Current² × Resistance |
| Power = V²/R | Power = Voltage² ÷ Resistance |
| Energy = Power × Time | Watt-hours = Watts × Hours |

## Resistors vs Capacitors vs Batteries — don't mix these up

| Component | Series | Parallel |
| --- | --- | --- |
| Resistors | R total = R1 + R2 (adds) | 1/R total = 1/R1 + 1/R2 (combines smaller) |
| Capacitors | C total = (C1×C2)/(C1+C2) | C total = C1 + C2 (adds) |
| Batteries | Voltage adds, capacity (Ah) stays | Capacity (Ah) adds, voltage stays |

COMMON MIX-UP Resistors and capacitors follow OPPOSITE patterns to each other in series vs parallel. Batteries follow yet another pattern (Section 3). This table is worth memorizing on its own.

# 8. Communication Protocols in an EV

The BMS, motor controller, and other electronic systems all need to talk to each other. Common protocols:

- CAN (Controller Area Network) — the classic automotive communication bus

- Modbus — common in solar/industrial systems

- Ethernet

- Wireless

- Bluetooth

# 9. Battery Aging & Degradation

| Type | Cause |
| --- | --- |
| Calendar Aging | Just from storage over time — worsened by high temperature and high state of charge |
| Cyclic Aging | From repeated charge/discharge cycles |

## Mechanisms behind aging

- Lithium plating — lithium metal coats the electrode instead of properly absorbing into it, reducing usable capacity

- SEI Growth (Solid Electrolyte Interphase) — a layer builds up inside the cell, increasing internal resistance over time

## Battery lifecycle stages, by capacity

| 100% – 80% SoH FIRST LIFE 5–8 yrs, in the EV | → | 80% – 60% SoH SECOND LIFE 5–10 yrs, stationary storage | → | < 60% SoH RECYCLING |
| --- | --- | --- | --- | --- |

WHY THIS MATTERS FOR YOUR WORK This 80%–60% SoH window is exactly the space where repurposed EV batteries for solar cold-storage systems operate — it's the second-life sweet spot.

# 10. Battery Assessment — The 10 Assessment Areas

FOCUS SECTION This is the core process to know cold for the assessment: a systematic 10-step evaluation used to determine a used battery's condition and residual value.

| 01 | Receive Battery & Document Review Documentation Check mileage, ID, recalls, and pull the BMS data logs. |
| --- | --- |

| 02 | Visual Inspection Physical Look for physical damage, swelling, corrosion, or leaks. |
| --- | --- |

| 03 | Voltage Measurement Electrical Measure cell and pack voltage against expected nominal values. |
| --- | --- |

| 04 | Insulation Testing Safety Check for voltage leakage — confirms HV isolation from chassis. |
| --- | --- |

| 05 | Capacity Testing Performance Discharge test to measure actual usable Ah/Wh. Watch for deep discharge, cell imbalance, and cell failure. Typical rates: 1C (1 hr) or ⅓C (3 hrs). |
| --- | --- |

| 06 | Internal Resistance Performance Lower internal resistance = healthier cell. |
| --- | --- |

| 07 | Temperature Monitoring Safety Track thermal behavior during charge/discharge under load. |
| --- | --- |

| 08 | Cell Balancing Electrical Check cell-to-cell voltage variance — target under 30mV difference. |
| --- | --- |

| 09 | BMS Diagnostics Documentation Pull fault/diagnostic trouble codes (DTCs) directly from the BMS. |
| --- | --- |

| 10 | State of Health (SoH) Assessment Outcome Final output: SoH% = (Current Capacity ÷ Rated Capacity) × 100. This determines residual value and next-life pathway. |
| --- | --- |

## Assessment areas grouped by category

| Category | Steps covered |
| --- | --- |
| Documentation | 1. Document review · 9. BMS diagnostics |
| Physical | 2. Visual inspection |
| Electrical | 3. Voltage measurement · 8. Cell balancing |
| Safety | 4. Insulation testing · 7. Temperature monitoring |
| Performance | 5. Capacity testing · 6. Internal resistance |
| Outcome | 10. State of Health → Residual Value |

THE ONE FORMULA TO KNOW COLD SoH (%) = (Current Capacity ÷ Rated Capacity) × 100. This single number is the output of the entire 10-step process, and it's what determines whether a battery stays in first-life EV use, moves to second-life storage, or is recycled.

# 11. Battery Circularity — The Full Lifecycle

| 1. Raw Material Extraction | → | 2. Battery Manufacturing | → | 3. Battery Integration | → | 4. Vehicle Operation |
| --- | --- | --- | --- | --- | --- | --- |

↓

| 5. Battery Assessment | → | 6. 2nd Life Application (5–10 yrs) | → | 7. Recycling (WEEE, EnviroServe) | → | 8. Recovery of Raw Materials |
| --- | --- | --- | --- | --- | --- | --- |

↓

9. Manufacturing of New Batteries — the loop closes

## Where raw materials come from

| Material | Main source region |
| --- | --- |
| Lithium | South America, Australia |
| Cobalt | Congo |
| Nickel | Sulphide ores |
| Graphite | China |

## Three circularity strategies — in order of value retained

| Priority | Strategy | What it means |
| --- | --- | --- |
| 1st (best) | Reuse | Use the battery as-is |
| 2nd | Repurpose | Give it a second life in a different application (e.g. stationary storage) |
| 3rd | Recycle | Recover raw materials once no longer usable at all |

## End-of-life signals

- Capacity loss below 80%

- Increased internal resistance (over 150% of original)

- Safety concerns (leakage)

- Manufacturer recommendations

# Quick-Reference Formula Sheet

| Concept | Formula |
| --- | --- |
| Ohm's Law | V = IR |
| Power | P = VI = I²R = V²/R |
| Energy | Wh = Power (W) × Time (h) |
| Battery energy | Wh = Ah × V |
| C-rating | C = Current drawn ÷ Capacity (Ah) |
| Energy density (gravimetric) | Wh ÷ kg |
| Energy density (volumetric) | Wh ÷ volume |
| State of Health | SoH% = (Current Capacity ÷ Rated Capacity) × 100 |
| Resistors — series | R total = R1 + R2 |
| Resistors — parallel | R total = (R1×R2)/(R1+R2) |
| Capacitors — parallel | C total = C1 + C2 |
| Capacitors — series | C total = (C1×C2)/(C1+C2) |
| Batteries — series | Voltage adds, capacity (Ah) stays same |
| Batteries — parallel | Capacity (Ah) adds, voltage stays same |

# The One-Page Mental Model

REMEMBER THIS IF NOTHING ELSE 1. Energy is stored in cells → grouped into modules → grouped into packs. 2. Series wiring adds voltage; parallel wiring adds capacity — but total stored energy (Wh) is identical either way. 3. The BMS is the brain, guarding against thermal runaway, imbalance, and tracking health. 4. Batteries age via calendar aging (storage) and cyclic aging (use) — tracked through State of Health. 5. A battery's working life doesn't end at 80% SoH — it moves from powering a vehicle to a second-life stationary application, and is only recycled once it drops below ~60%. 6. The 10-step assessment (Section 10) is how all of the above gets measured and turned into one number: SoH — and that's the number that decides everything downstream.